# Scotian Shelf Climatology Report

Emily O’Grady

February 2025

# Introduction

This report details how a climatology was generated to update the quality control procedures used at Bedford Institute of Oceanography. This work is based off a similar project led by Gordana Lazin in 2014. The goal of this project was to update her climatology to reflect a more recent time period to maintain the quality control procedures. This will contribute to the quality of future BioChem records.

# Data Distribution

Data was extracted from BioChem in the same geographic range as Lazin 2014, 37° N to 48° N and 71° W to 48° W. The temporal range of the extraction was limited from 1999-2023 (which was at the time the most recent data available). This is significantly different from the previous version of the climatology which was not temporally limited and included all historical records in Biochem, back to the 1950s.

Depth bins were defined at 4 intervals, and range values were calculated for 0-10m, 10-30m, 30-100m and 100m to a maximum depth of 5000m (Brickman and Petrie 2003).

# Quality Control

Since there has been a large cleanup effort on BioChem data, and data post 1999 is much less prone to metadata errors, the quality control required for this version of the climatology was slightly less involved than the previous version. The general quality checks on data were the same.

- Check methodology

Each parameter contains multiple methods, and so a check was performed to ensure that all methods were comparable. This mostly impacted oxygen data as other parameter’s had been sampled under all comparable methods. Oxygen data was filtered to include only Winkler titration data records.

- Check units

It was confirmed that all units were identical within each parameter

- Metadata checks

Depth and position metadata were checked with no major issues detected. This included checking for 0 values in position information, negative depth values, checking date ranges, as well as time and date QC codes

- Data checks

  - Quality control flags

    - Flags of 0, 1, 2, 3, 6 were included in climatology calculations. This excluded only data points with flags of 4 (erroneous), and 5 (which was used sparsely in historical data to denote a manual modification was made)

  - Coastal exclusion

    - Data within 5km of the coast was excluded from climatology calculations due to major shifts in nutrient regimes in near-shore areas

  - Climatology boxes (Figure 1)

    - Data outside the defined boxes was also excluded from the climatology

Each individual variable was then run through range checks, including global and regional ranges which were defined based on WOD23 data values (Appendix I). Data outside these expected ranges was excluded from the climatology product. There were also three missions (CAR2021221, BCD2005667, BCD2019666) which were manually filtered out due to errors in the BioChem database, which are awaiting resolution.

The following sections detail the results of these QC processes on each variable including the number of data points excluded based on each test, as well as the temporal and spatial range of the data.

Figure 1. This shows the boxes used for the Scotian Shelf climatology. They are based on Petrie boxes (Petrie 1996), modified by Andrew Cogswell (2014). Figure from Lazin 2014.

# Parameter Details

## Ammonia

Ammonia was not previously included in the climatology product, therefore it has not undergone consistent QC through the last 10 years. There have been some flags applied to the data from lab insight and QC efforts. Ammonia also does not currently have any global, regional, or profile range values in our QC methodology. This dataset could benefit from some additional QC. The inclusion of this variable in the updated climatology will allow us to QC the entire nutrient datasets produced from AZMP missions, and increase the coverage of our QC methods. Ammonia methods included fresh, frozen and undefined protocols.

| Ammonia 2024 | Data removed | Total data |
| --- | --- | --- |
| extracted |  | 77770 |
| data flags | 1493 |  |
| spatial filters (coastal buffer+boxes) | 7824 + 13053 |  |
| Test 2.1 (global range) | NA |  |
| Test 2.2 (regional range) | NA |  |
| Test 2.4 (profile envelope) | NA |  |
| manual filtering for errors | 46 |  |
|  |  | 55358 |

## Nitrite

Nitrite was not included in the previous version of the climatology, although it has been run through stage 2 of the QC methodology over the last 10 years, so it has undergone partial QC based on expected global, regional and profile ranges. The inclusion of this variable in the updated climatology will help increase the coverage of QC for AZMP nutrient data and improve the quality of future nutrient datasets. Nitrite methods included in this dataset were fresh, frozen, super frozen and undefined protocols.

| Nitrite 2024 | data removed | total data |
| --- | --- | --- |
| extracted |  | 91537 |
| data flags | 434 |  |
| spatial filters (coastal buffer+boxes) | 9186+21569 |  |
| Test 2.1 (global range) | 0 |  |
| Test 2.2 (regional range) | 0 |  |
| Test 2.4 (profile envelope) | 0 |  |
| manual filtering for errors | 41 |  |
|  |  | 60356 |

## Oxygen

Oxygen data was not previously included in the climatology product, although oxygen data has undergone stage 2 QC over the last 10 years. Oxygen data has been thoroughly checked by stage 2 tests including global, regional and profile ranges as well as comparative CTD- Bottle checks. Some initial filtering was done to isolate oxygen methodologies and units, this resulted in a smaller initial dataset. Only Winkler data was used to calculate the climatology, due to the lack of confidence in other methods. Oxygen data also currently undergoes additional checks in the QC system including a oxygen saturation test for surface data. Adding this variable to the climatology will ensure the highest quality oxygen data moving forward.

| O2 2024 | Data removed | Total data |
| --- | --- | --- |
| extracted |  | 17829 |
| data flags | 203 |  |
| spatial filters (coastal buffer+boxes) | 1509+919 |  |
| Test 2.1 (global range) | 0 |  |
| Test 2.2 (regional range) | 0 |  |
| Test 2.4 (profile envelope) | 1 |  |
| manual filtering for errors | 0 |  |
|  |  | 15198 |

## Phaeophytin

Phaeophytin data was not previously included in the climatology product for the Scotian Shelf. The methods available included Holm-Hansen fresh and frozen, as well as some undefined protocol data. This data set does not currently have any global or regional ranges for the Scotian Shelf and so no data was filtered out through Test 2 QC before inclusion in the climatology. Some flags have been applied by data producers.

| Phaeophytin 2024 | Data removed | Total data |
| --- | --- | --- |
| extracted |  | 72592 |
| data flags | 71 |  |
| spatial filters (coastal buffer+boxes) | 5980+11137 |  |
| Test 2.1 (global range) | NA |  |
| Test 2.2 (regional range) | NA |  |
| Test 2.4 (profile envelope) | NA |  |
| manual filtering for errors | 44 |  |
|  |  | 55378 |

## Nitrate

Nitrate data was previously included in the climatology product, which allows us to compare our updated values to those calculated in 2014. The methods in this dataset included fresh, frozen and super frozen as well as undefined protocol data. These data have been undergoing through QC over the last 10 years, including climatology and expected range tests, so this is a very high quality dataset. Despite a significant shift in temporal restrictions between the 2014 and 2025 product, the final number of records used to calculate the climatology are quite close due to the increase an increase in available data post 2014.

| extracted | 109239 |
| --- | --- |
| Ocean data within all sub-areas | 72 000 |
| Data Flagged | 301 |
| Position Flagged | 0 |
| Exceeding upper limit (515 mmol/m3) | 0 |
| Negative | 1 |
| Total removed | 302 |
| Filtered dataset | 71 698 |

Table 1 Number of nitrate records identified during quality control. [Lazin 2014]

| Nitrate 2024 | Data removed | Total data |
| --- | --- | --- |
| extracted |  | 110023 |
| data flags | 452 |  |
| spatial filters (coastal buffer+boxes) | 11093 + 24572 |  |
| Test 2.1 (global range) | 3 |  |
| Test 2.2 (regional range) | 0 |  |
| Test 2.4 (profile envelope) | 0 |  |
| manual filtering for errors | 64 |  |
|  |  | 73926 |

Figure 1 Depth profile of all nitrate data used for the climatology computation. Nitrate unit is mmol/m3 [Lazin 2014]

Figure 2 Depth profile of all nitrate data used for 2024 climatology [1999-2024]. Unit is mmol/m3.

## Silicate

Silicate was previously included in the climatology product, and has been confidently quality controlled over the past 10 years. This has included both climatology and global, regional and profile range checks. Similar to nitrate, this data set has an equivalent number of data points used to generate the climatology to the 2014 product, despite the temporal restrictions, due to an increase in data availability and sampling frequency. Silicate protocols included fresh, frozen and super frozen as well as some undefined protocol data.

Table 2  Number of silicate records identified during quality control. [Lazin 2014]

| Extracted | 115047 |
| --- | --- |
| Ocean data within all sub-areas | 73 387 |
| Data Flagged | 1876 |
| Position Flagged | 0 |
| Exceeding upper limit | 0 |
| Negative | 0 |
| Outside envelope | 12 |
| Total removed | 1888 |
| Filtered dataset | 71 499 |

| Silicate 2024 | Data removed | Total data |
| --- | --- | --- |
| extracted |  | 109164 |
| data flags | 411 |  |
| spatial filters (coastal buffer+boxes) | 11009 + 24104 |  |
| Test 2.1 (global range) | 0 |  |
| Test 2.2 (regional range) | 0 |  |
| Test 2.4 (profile envelope) | 0 |  |
| manual filtering for errors | 64 |  |
|  |  | 73663 |

Figure 3 Depth profile of all silicate data used for the climatology computation. Silicate unit is mmol/m3. [Lazin 2014]

Figure 4 Depth profile of all silicate data used for 2024 climatology [1999-2024]. Unit is mmol/m3.

## Phosphate

Phosphate was previously included in the climatology product, and has been confidently quality controlled over the past 10 years. This has included both climatology and global, regional and profile range checks. Similar to nitrate, and silicate this data set has an equivalent number of data points used to generate the climatology to the 2014 product, despite the temporal restrictions, due to an increase in data availability and sampling frequency. There are slightly more data points flagged in this dataset compared to other nutrients due to the tendency of phosphate to be sensitive to contamination. This dataset specifically is vulnerable to producing poor replicate measurements under certain conditions, and is generally more sensitive to data quality issues. Phosphate methods included fresh, frozen and super frozen as well as some undefined protocol data.

Table 3 Number of phosphate records identified during quality control. [Lazin 2014]

| extracted | 121630 |
| --- | --- |
| Ocean data within all sub-areas | 75 250 |
| Data Flagged | 728 |
| Position Flagged | 0 |
| Exceeding upper limit | 2 |
| Negative | 0 |
| Outside envelope | 6 |
| Total removed | 736 |
| Filtered dataset | 74 514 |

| Phosphate 2024 | data removed | total data |
| --- | --- | --- |
| extracted |  | 109722 |
| data flags | 511 |  |
| spatial filters (coastal buffer+boxes) | 11064 + 24553 |  |
| Test 2.1 (global range) | 0 |  |
| Test 2.2 (regional range) | 0 |  |
| Test 2.4 (profile envelope) | 0 |  |
| manual filtering for errors | 58 |  |
|  |  | 73623 |

Figure 5 Depth profile of all phosphate data used for the climatology computation. Phosphate unit is mmol/m3. [Lazin 2014]

Figure 6 Depth profile of all phosphate data used for 2024 climatology [1999-2024]. Unit is mmol/m3.

## Chlorophyll A

Chlorophyll A was included in the 2014 climatology product, and has been confidently quality controlled over the last 10 years, producing a high quality dataset. These quality control checks have included stage 2 QC comparison to global, regional and profile ranges, as well as stage 3 climatology range tests. This dataset includes Welschmeyer and Holm-Hansen fresh, frozen and super frozen protocols, as well as some undefined protocol data.

Table 4  Number of chlorophyll records identified during quality control. [Lazin 2014]

| Extracted | 89799 |
| --- | --- |
| Ocean data within all sub-areas | 59 407 |
| Data Flagged | 983 |
| Position Flagged | 0 |
| Exceeding upper limit | 0 |
| Negative | 0 |
| Total removed | 983 |
| Filtered dataset | 58 424 |

| Chlorophyll A 2024 | Data removed | Total data |
| --- | --- | --- |
| extracted |  | 92187 |
| data flags | 314 |  |
| spatial filters (coastal buffer+boxes) | 10791 + 22989 |  |
| Test 2.1 (global range) | 0 |  |
| Test 2.2 (regional range) | 0 |  |
| Test 2.4 (profile envelope) | 0 |  |
| manual filtering for errors | 58 |  |
|  |  | 58099 |

Figure 7: Depth profile of all chlorophyll data used for the climatology computation. Chlorophyll unit is mg/m3. [Lazin 2014]

Figure 8 Depth profile of all chlorophyll A data used for 2024 climatology [1999-2024]. Unit is mg/m3.

# Climatology Computation

The climatology calculations included monthly mean, minimum and maximum values for each individual box and depth bin combination. Due to data availability, there were cases were there were not enough data points (n < 2) in a specific area, depth and month. In these cases, if there was data available in the 2014 climatology, those values were merged into the final climatology product. These cases were rare and represent less than 10% of the climatology.

The final climatology product has been exported as a csv from the R processing workflow. The workflow can be found on GitHub (AZMP_climatology). Once the csv was exported, it was also reformatted to fit the MatLab structure of the current QC system.

Details of data processing can be found in the README on the GitHub repository’s homepage.

# References

Brickman, D. and B. Petrie. 2003. Nitrate, silicate and phosphate atlas for the Gulf of St. Lawrence. Can. Tech. Rep. Hydrogr. Ocean Sci. 231, xi+152 pp.

IML quality control procedure for bottle data:

http://slgo.ca/app-sgdo/en/docs_reference/botl_odf_quality.html

DFO (2014). BioChem: database of biological and chemical oceanographic data. Department of Fisheries and Oceans, Canada.http://www.meds-sdmm.dfo-mpo.gc.ca/biochem/biochem-eng.htm.

Devine, L., M.K. Kennedy, I. St-Pierre, C. Lafleur, M. Ouellet, and S. Bond. 2014. BioChem: the Fisheries and Oceans Canada database for biological and chemical data. Can. Tech. Rep. Fish. Aquat. Sci. 3073: iv + 40 pp. http://www.dfo-mpo.gc.ca/Library/351319.pdf

# Appendix I

Tables of Global and regional ranges used to QC data before inclusion in climatology

Test 2.1 – Global Range

| Variable | Unit | Min | Max |
| --- | --- | --- | --- |
| Salinity | (psu) | 0 | 40 |
| Dissolved oxygen | mL/L | 0 | 11 |
| Chlorophyll | mg/m³ | 0 | 50 |
| Phaeophytin | mg/m³ | NA | NA |
| Nitrate+Nitrite | mmol/m³ | 0 | 500 |
| Nitrite | mmol/m³ | 0 | 15 |
| Ammonia | mmol/m³ | NA | NA |
| Phosphate | mmol/m³ | 0 | 60 |
| Silicate | mmol/m³ | 0 | 250 |

Test 2.2 – Regional Range

| Variable | Unit | Min | Max |
| --- | --- | --- | --- |
| Salinity | (psu) | 5 | 40 |
| Dissolved oxygen | mL/L | 0 | 10 |
| Chlorophyll | mg/m³ | 0 | 50 |
| Phaeophytin | mg/m³ | NA | NA |
| Nitrate+Nitrite | mmol/m³ | 0 | 48 |
| Nitrite | mmol/m³ | 0 | 15 |
| Ammonia | mmol/m³ | NA | NA |
| Phosphate | mmol/m³ | 0 | 4.5 |
| Silicate | mmol/m³ | 0 | 250 |

Test 2.4 – Profile Envelopes

| Variable | Min Depth | Max Depth | Min Value | Max Value |
| --- | --- | --- | --- | --- |
| phosphate | 0 | 500 | 0 | 4.5 |
| phosphate | 500 | 5500 | 0.01 | 4.5 |
| silicate | 0 | 200 | 0 | 150 |
| silicate | 200 | 1000 | 0.01 | 150 |
| silicate | 1000 | 1500 | 2.5 | 150 |
| silicate | 1500 | 4500 | 5 | 150 |
| silicate | 4500 | 5500 | 10 | 150 |
| nitrate | 0 | 50 | 0 | 18 |
| nitrate | 50 | 200 | 0 | 30 |
| nitrate | 200 | 300 | 0 | 34 |
| nitrate | 300 | 700 | 0 | 42 |
| nitrate | 700 | 1100 | 6 | 46 |
| nitrate | 1100 | 4000 | 6 | 48 |
| nitrate | 4000 | 5500 | 10 | 46 |
| chlorophyll | 0 | 20 | 0 | 3 |
| chlorophyll | 20 | 50 | 0 | 2.5 |
| chlorophyll | 50 | 100 | 0 | 2 |
| chlorophyll | 100 | 250 | 0 | 0.5 |
| chlorophyll | 250 | 2500 | 0 | 0.4 |
| chlorophyll | 2500 | 3000 | 0 | 0.3 |
| chlorophyll | 3000 | 5500 | 0 | 0.05 |
| temperature | 0 | 10 | -2.1 | 35 |
| temperature | 10 | 50 | -2.1 | 32 |
| temperature | 50 | 100 | -2.1 | 30 |
| temperature | 100 | 300 | -2 | 28 |
| temperature | 300 | 1000 | -1.7 | 20 |
| temperature | 1000 | 1500 | -1.5 | 18 |
| temperature | 1500 | 3000 | -1.5 | 13 |
| temperature | 3000 | 5500 | -1.5 | 7 |
| salinity | 0 | 10 | 5 | 40 |
| salinity | 10 | 50 | 27 | 38.2 |
| salinity | 50 | 125 | 28.9 | 38 |
| salinity | 125 | 300 | 29.4 | 37.6 |
| salinity | 300 | 600 | 30.8 | 36.8 |
| salinity | 600 | 2000 | 32.2 | 36.6 |
| salinity | 2000 | 5500 | 34.7 | 35.5 |
| Oxygen | 0 | 30 | 0 | 10 |
| Oxygen | 30 | 200 | 0 | 9 |
| Oxygen | 200 | 1500 | 0 | 8 |

# References

IML quality control procedure for bottle data:

http://slgo.ca/app-sgdo/en/docs_reference/botl_odf_quality.html

Petrie, B., P. Yeats and P. Strain. 1999. Nitrate, silicate and phosphate atlas for the Scotian Shelf and Gulf of Maine. Can. Tech. Rep. Hydrog. Ocean Sci. 203, vii + 96 pp.

DFO (2024). BioChem: database of biological and chemical oceanographic data. Department of Fisheries and Oceans, Canada.http://www.meds-sdmm.dfo-mpo.gc.ca/biochem/biochem-eng.htm.

Devine, L., M.K. Kennedy, I. St-Pierre, C. Lafleur, M. Ouellet, and S. Bond. 2014. BioChem: the Fisheries and Oceans Canada database for biological and chemical data. Can. Tech. Rep. Fish. Aquat. Sci. 3073: iv + 40 pp. http://www.dfo-mpo.gc.ca/Library/351319.pdf